10.02Commercial and Industrial Noise

B.  MAXIMUM PERMISSIBLE SOUND PRESSURE LEVELS

The maximum permissible sound pressure levels at or beyond the emitting property boundary shall not exceed the levels in Table 1.

TABLE 1: Sound Levels at Receiving Property

| Time | Sound Level Limit (dBA LFmax) | Sound Level Limit (dBC LFmax) |
| --- | --- | --- |
| 7 a.m.- 10 p.m. 10 p.m. - 7 a.m. | 55 dBA 45 dBA | 65 dBC 55 dBC |

C. NIGHTTIME NOISE

1.  In addition to the levels prescribed in Table 1, noise between 10 p.m. and 7 a.m. shall not exceed 37 dBA Leq(15min.) or 47 dBC Leq(15min.) at or beyond the property boundary.

2. Noise from non-residential uses, such as a home business shall not be plainly audible within a residential structure between 10 p.m. and 7 a.m.

D. EXEMPTIONS

The sound level limits in this Article shall not apply to:

- Snow removal,

- Construction, renovation, or demolition activities with a valid town permit between 7 a.m. - 6 p.m.,

- Lawn and property maintenance between 7 a.m. - 10 p.m.,

- Vehicles while accessing the property (vehicles while crossing the property boundary),

- Town or school sponsored events or parades,

- Noise resulting from emergency work in order to protect property or human safety,

- Noise permitted by a temporary Noise Permit, which has been granted by the Board or Selectmen under the Town of Bethlehem Noise Ordinance which is valid for three months,

- Noise resulting from agricultural activity exempted under RSA 31:39, paragraph n.

E. DEFINITIONS

A-WEIGHTED SOUND LEVEL — The sound pressure level measured by the use of an instrument with the metering characteristics and the A-weighting frequency response prescribed for sound level meters by ANSI S1.4-1983 or its successors. The level so read is designated "dBA."

C-WEIGHTED SOUND LEVEL — The sound pressure level measured by the use of an instrument with the metering characteristics and the C-weighting frequency response prescribed for sound level meters by ANSI S1.4-1983 or its successors. The level so read is designated "dBC."

DECIBEL (dB) — The practical unit of measurement for sound pressure level; the number of "decibels" of a measured sound is equal to 20 times the logarithm to the base 10 of the ratio of the sound pressure of the measured sound to the sound pressure of a reference sound of 20 micropascals; abbreviated "dB."

dBA LF(max.) — The maximum quantity (over a measurement period) in decibels measured by a sound-level meter satisfying the requirements of American National Standards specification for sound-level meters with standardized dynamic characteristic "fast" and A-weighting.

dBC LF(max.) — The maximum quantity (over a measurement period) in decibels measured by a sound-level meter satisfying the requirements of American National Standards specification for sound-level meters with standardized dynamic characteristic "fast" and C-weighting.

dBA Leq(15 min.)  — The 15 minute Leq (Level Equivalent) or “average” sound level in decibels measured by a sound-level meter satisfying the requirements of American National Standards specification for sound-level meters with standardized Leq capability and A-weighting.

dBC Leq(15 min.) — The 15 minute Leq (Level Equivalent) or “average” sound level in decibels measured by a sound-level meter satisfying the requirements of American National Standards specification for sound-level meters with standardized Leq capability and C-weighting.

MEASUREMENT PERIOD – The continuous time interval over which sound is measured, which shall be fifteen (15) minutes.

PLAINLY AUDIBLE — Any sound that can be detected by a person using his or her unaided hearing faculties. As an example, if the sound source under investigation is from a commercial establishment such as a home business, restaurant, bar, or outdoor event, the Selectboard or its designee need not determine the title of a song or be able to understand specific words,. The detection of the rhythmic bass component of the music is sufficient to constitute a plainly audible sound.

SOUND LEVEL — The quantity in decibels measured by a sound-level meter satisfying the requirements of American National Standards specification for sound-level meters. "Sound level" is the frequency weighted sound pressure level obtained with the standardized dynamic characteristic "fast" or “Leq” and weighting (A) or (C).

SOUND-LEVEL METER — An instrument, including a microphone, an amplifier, an output meter and frequency waiting networks, for the measurement of sound levels.